AtlasIED FC Series BIM file Usage Notes

The Atlas-IsoFlare FC Ceiling Mount Series Assembly_2019-rev1 Family is an “assembly” Family which includes all models of the FC series in white and black and the associated mounting hardware and optional grilles. The Family is “Face Based” and can be inserted on any Face in your project, including faces of any linked models. The zip file includes the Family and a text file with the same name. This text file must be saved in the same location as the FC BIM file in order for the library function to work correctly.

When you first load the Family into your project, you will see a dialog box listing all the available models (Types) in the Family along with all the data for each type.

The user may select any combination of Types desired for the project. This may be just one or all, depending on your needs. This helps to reduce the file size of your project model. Once loaded, you have the option to insert the Family on a “Vertical Face, “Face” or “Workplane”. As these are ceiling loudspeakers, you would typically select the “Face” option.

Note that the Family will show up under “Communication Devices” in your project “Families” Browser. You will also find nested Hardware Families under “Generic Models”. Please avoid using these Generic models FC these brackets are already included in the Communications Devices Family. Also note that you may create Schedules such as a BOM listing all models in your project along with the correct mounting hardware if they are optional items that must be ordered separately.

We have included the AVIXA standard loudspeaker symbol which will be visible when a View is set to “Coarse” or “Medium”. The 3D model of the loudspeaker is set to be visible in the “Medium” and “Fine” settings.

Detail Level:

“Coarse”			        “Medium”			               “Fine”

The Family also includes industry standard spacing and REP (Required Electrical Power) calculations to calculate the standard Minimum Overlap and Edge-to-Edge spacing distances, and the minimum amplifier power needed to achieve the level needed at the listener.

After inserting a loudspeaker in your model, click to select the loudspeaker to see it’s properties (per instance). Look in the “Constraints” section under the “Properties” window to see the “Elevation” of the Face that is hosting the loudspeaker and enter this value into the “Ceiling Height” parameter under the “Model Properties” section, then enter the desired “Listener Height” (i.e., standing/seated height) parameter and you will see the calculated spacing distance for both Edge-to-Edge and Minimum Overlap spacing patterns.

Note: The user may wish to alter the “Dispersion” value based on their own interpretation of the polar plots or beamwidth plots. This is not recommended for users that are not experienced in reading and understanding this data as it alters the results of the spacing calculations.

There are also graphics representing these distances that may be turned on/off under the “Edit Type” tab. This will show a circle with the diameter of the selected spacing distance giving the user the option to visually copy and paste multiple loudspeakers where needed while maintaining the proper distances for the desired coverage pattern.

This is a “Type” parameter, so the graphics will be turned on/off on all loudspeakers of the same model in the entire Project.

For amplifier power requirements, enter the level needed in the “Desired Level @ Listener” parameter and the desired “Peak Factor”. The Family will then calculate the minimum power required in the “REP” parameter and select the appropriate available tap (per model) needed to achieve the target level at the listener.

There are also switch parameters allowing the user to select the hardware and grille options are needed for each instance.